Lab 2

| Author | Name：冯柏钧 Student ID:12011124 Name：周安然 Student ID:12011214 |
| --- | --- |
| Introduction The lab guide us to learn about inverse filtering of LTI systems through the example of echo cancelling. We will load the given file and use MATLAB to process them and discuss the properties of LTI systems. Lab results & Analysis： 2.10 Question(a) Analysis From the description, we can know that the result is as shown. Question(b) Analysis From y[n] = x[n] + ax[n - N](2.21) and z[n] + az[n - N] = y[n](2.22), we can derive that x[n] + ax[n - N] = z[n] + az[n - N] is true for all possible n, which means is apparent that z[n] = x[n] is a valid solution to the overall difference equation. Moreover, z[n] * he[n] = y[n]. Question(c) The result is as shown. Question(d) The result is as shown and the new output has no echo. Question(e) Analysis As the figure shown, the result is not an unit impulse. The reason is that the echo removal system has an infinite-length impulse response while he and her are both finite. There exists difference. Question(f) Analysis 𝑅𝑦𝑦 [𝑛] = 𝑥 [𝑛] ∗ (𝛿 [𝑛] + 𝛼𝛿 [𝑛 – 𝑁]) ∗ 𝑥 [−𝑛] ∗ (𝛿 [𝑛] + 𝛼𝛿 [𝑛 + 𝑁])= = 𝑅𝑥𝑥 ∗ ((1 + 𝛼2)𝛿 [𝑛] + 𝛼𝛿 [𝑛 – 𝑁] + 𝛼𝛿[𝑛 + 𝑁]) Figure for y2 N should be 3123 Figure for y3 N1 should be 2808 and N2 should be 2197. Note: Please indicate meaning of the symbols in all expressions. Please indicate the coordinate and unit in all figures. |  |
| Experience 冯柏钧 |  |
| Score |  |

Code

2.10

Question(a)

he = [1 zeros(1,999) 1.5];

nhe = 0:1000;

stem(nhe, he);

xlabel('index');

ylabel('value');

title('he');

Question(c)

clc

clear

load lineup.mat

%sound(y,8192);

N = 1000;

he = [1 zeros(1,999) 1.5];

nhe = 0:1000;

d = [1 zeros(1,4000)];

A = [1 zeros(1,N-1), 0.5];

B = 1;

her = filter(B,A,d);

nher = 0:4000;

stem(nher, her);

xlabel('index');

ylabel('value');

title('her');

Question(d)

clc

clear

load lineup.mat

%sound(y,8192);

N = 1000;

he = [1 zeros(1,999) 1.5];

nhe = 0:1000;

d = [1 zeros(1,4000)];

A = [1 zeros(1,N-1), 0.5];

B = 1;

her = filter(B,A,d);

a = A;

z = filter(1,a,y);

plot(z);

xlabel('index');

ylabel('value');

title('z');

sound(z, 8192);

Question(e)

clc

clear

load lineup.mat

%sound(y,8192);

N = 1000;

he = [1 zeros(1,999) 1.5];

nhe = 0:1000;

d = [1 zeros(1,4000)];

A = [1 zeros(1,N-1), 0.5];

B = 1;

her = filter(B,A,d);

a = A;

z = filter(1,a,y);

hoa = conv(he,her);

plot(hoa);

xlabel('index');

ylabel('value');

title('hoa');

Question(f)

clc

clear

load lineup.mat

Ryy = conv(y,fliplr(y));

plot(Ryy);

xlabel('index');

ylabel('value');

title('Ryy');

clc

clear

load lineup.mat

Ryy2 = conv(y2,fliplr(y2));

plot(Ryy2);

xlabel('index');

ylabel('value');

title('Ryy2');

clc

clear

load lineup.mat

Ryy3 = conv(y3,fliplr(y));

plot(Ryy3);

xlabel('index');

ylabel('value');

title('Ryy3');